Lab 3			Name _______________________________________

MCSE 1

Objective:

-  Configure Windows 7, Windows 8, and Windows 10 clients on various networks to

practice network principals.

Procedure:

___	Make sure Ottawa, Hamilton, and Saskatoon are connected to VMNet2.

Single-click

Set to VMnet2

Single-click

Fig. 1    Connecting the virtual machine to VMNet2

___	Make sure all 3 virtual machines are set for at least 1 GB.  You can change the 	amount of RAM allocated to virtual machine the same way you set the VMNet 	connection.

___	Start Ottawa, Hamilton and Saskatoon.

Your network at home has a router that acts like a DHCP server that will automatically give your computers IP addresses.  The network in the lab is just three computers connected to a switch.  There is no DHCP server to give your computer an IP address.

Saskatoon:

___	Configure the network card on Saskatoon to obtain a DHCP address.  To do this, right-click on Network icon in the task bar and select Open Network and Sharing Center.  Click on the Ethernet0 connection.  Configure the NIC as shown below.

Double-click on IPv4

These fields should be blank

Clear the box for IPv6

Click on Properties

Fig. 2 Configuring the NIC to obtain an IP address

Note:	There will be no screen captures for this lab.  You will answer all the 	questions on this printed copy of the lab and then submit the paper lab to 	be marked.

___	Click on OK a couple of times to close the properties page of the network card.

___	In the DOS window type ipconfig   /all.

1.	Record the following information:

MAC address (Physical Address): 	____________________________

IP Address					____________________________

Subnet Mask 				____________________________

2.	How many bits make up the network portion of the IP address?   _____

3.	What class of license is this?  	________

4.	How can you tell what class of license this is?

____________________________________________________________

5.	What is the network address?		____________________________

The client tried to obtain an IP address from the server.  There is no server so the client gave itself an APIPA (Automatic Private IP Address) address.

Ottawa:

___ 	Configure Ottawa so it will try to obtain a DHCP address.  Follow the instructions

You used for Windows 10.

6.	Record the following information:

MAC address (Physical Address): 	____________________________

IP Address					____________________________

Subnet Mask 				____________________________

7.	How many bits make up the network portion of the IP address?   _____

8.	What class of license is this?  		_________

9.	What is the network address?		____________________________

10.	Will Ottawa and Saskatoon be able to communicate with each other?  	Explain why you think they will or will not.

___	Ping Ottawa from Saskatoon.  You should be successful.

___	Ping Saskatoon from Ottawa.  You will probably be unsuccessful.

Windows 10 on Saskatoon may have its firewall on.  This blocks the ping command.

___	On Saskatoon, turn the firewall off by following these steps.

___	Open the Control Panel.  Select System and Security.  Scroll to the bottom of the Security Center window.  Click on Windows Firewall.  Select Turn Windows Firewall on or off.

Fig. 3  Turning the firewall off

___	Try pinging Saskatoon from Ottawa, again.  You should be successful this time.

Hamilton:

___	Configure Hamilton so it tries to request an IP address from a DHCP server.

Right-click on the Network icon

Click on the Desktop tile

Fig. 4  Configuring the NIC on Windows 8

___	After clicking on the network icon in figure 4, the rest of the steps are the same 	as they were for Windows 10.  See figure 2.

___	Get back to the Start screen shown in figure 4 by pressing the windows key.

___	Click on the dark background of the desktop and type cmd.

___	Type ipconfig  /all.

11.	What is the IP address?		____________________________

12.	What is the network address?	____________________________

13.	Is Hamilton on the same network as Saskatoon and Ottawa?  ______

___	Ping Ottawa and Saskatoon.  You should be successful.

Saskatoon:

___	Configure Saskatoon to use an IP address of 10.1.1.1/16.

___	On Saskatoon, type ipconfig /all.

14.	What is the network address?	____________________________

15.	Is Saskatoon on the same network as Hamilton and Ottawa?  ______

Hamilton:

___	Ping Ottawa.

___	Ping Saskatoon.

16.	Record the results of the last 2 pings and explain why you got those 	results.

Ottawa:

___	Configure the NIC on Ottawa with an IP address of 10.1.2.2/16

Hamilton:

___	Configure the NIC on Hamilton with an IP address of 10.1.3.3/16

17.	What is the network address for Ottawa?    ________________________

18.	What is the network address for Hamilton? ________________________

___	From Hamilton, ping Saskatoon and Ottawa.  You should be successful both 	times.

___	On Hamilton change the subnet mask to 255.255.255.0.

___	From Hamilton, ping Saskatoon and Ottawa.  You should be unsuccessful both 	times.

19.	Why did changing the subnet mask cause Hamilton to not be able to 	ping Saskatoon and Ottawa?

___	Configure the 3 virtual machines for so they are using an IP address on a

private class B license.  Make sure they are all using the same network.

20.	What address did you use for Saskatoon?    _____________________

21.	What address did you use for Hamilton?  ________________________

22.	What address did you use for Ottawa?     ________________________

23.	What subnet mask did you use?               ________________________

24.	What network are the 3 computers on?    ________________________

___	Try pinging from Saskatoon to Ottawa and Hamilton.  You should be successful.

___	Configure the 3 virtual machines for so they are using an IP address on a

private class C license.  Make sure they are all using the same network.

25.	What address did you use for Saskatoon?     _____________________

26.	What address did you use for Hamilton?  ________________________

27.	What address did you use for Ottawa?     ________________________

28.	What subnet mask did you use?               ________________________

29.	What network are the 3 computers on?   ________________________

___	Try pinging from Saskatoon to Ottawa and Hamilton.  You should be successful.

___	Configure Saskatoon for 10.1.1.1/24.

___	Configure Ottawa for 10.1.1.2/24.

___	Configure Hamilton for 10.1.1.3/24

___	Ping to make sure you can communicate between the 3 hosts.

Windows 7 and Windows 8 Tips:  Pinning applications to the Taskbar

With Windows 7, 8 and 10, you can add or pin frequently used applications to the taskbar to make them readily available.

Ottawa:

___	Open a command prompt.

___	Notice the icon representing the window appears in the task bar.  To make it stay on the taskbar after the window is closed, right-click the icon and select pin this program to taskbar.

Fig. 5  Pinning the command prompt to the task bar

The same thing can be done with Windows 8.

___	From the start window on Hamilton, click on the dark background on the desktop 	and type cmd.

___	Right-click on the Command prompt box in the upper left corner and select pin to 	taskbar at the bottom of the screen.

___	Return to the Start window and click on the Desktop tile.

You should see the command prompt icon on the taskbar.

Ok….

You’re

done !!!!!!!