Lab 5			Name _________________________________________

MCSE 			      NetBIOS Name Resolution

Objective:

In this lab you will:

-  work with the NetBIOS cache

-  observe how broadcasts are used for local NetBIOS resolution

-  investigate the role of a WINS server when resolving names

-  configure the lmhosts file to statically configure NetBIOS names

Procedure:

_x__	Start Ottawa (Windows 7), Hamilton (Windows 8) and Calgary (Server 2008 R2).  Log into the Administrator’s account on each computer.

Calgary:

__x_	Put Calgary on VMNet 2 so it can communicate with Ottawa and Hamilton.

_x__	Configure the NIC for 10.1.1.10/16.  The preferred DNS server will be 	Calgary, 10.1.1.10.  There is no gateway.

_x__	Turn off IPv6.

_x__	Make sure you can ping by IP address between Ottawa, Hamilton and Calgary.

_x__	Start Wireshark on Hamilton.

Hamilton:

_x__	Make sure you are on the start screen.  Type CMD.

_x__	Right-click on the CMD box in the upper-left corner and select Run as Administrator in the menu bar at the bottom of the screen.

If you do not run cmd as the administrator, the arp –d command will fail.

__x_	Check the ARP cache by typing arp –a.  Clear the ARP cache by typing arp –d *.  Display the arp cache again.  (There is a multicast address of 224.0.0.22.  Just ignore it.  This is an IGMPv3 packet.)

1.	Capture the DOS window on Hamilton showing the output from the 3 commands;  arp –a, arp –d *, and arp –a again.  The ARP cache should be empty when the last command is executed.

_x__	On Hamilton, ping the IP address of Ottawa.

__x_	When the pings have finished, stop the Wireshark capture.

The window should look like figure 1.

Fig. 1  The wireshark capture on Hamilton when it pings the IP of Ottawa

Notice the first two frames are an ARP request and an ARP reply.

2.	Explain the purpose to the two ARP frames.  In your explanation use the computer names, not the IP addresses.

Once Hamilton has Ottawa’s MAC address, the first ping is sent to Ottawa.

Ottawa then ARPs for Hamilton’s MAC.  When Hamilton sends its MAC address, Ottawa sends the ping replay.

What happens if Hamilton pings Ottawa by name instead of IP address?  The name must be translated into an IP address.  This is the job of DNS or NetBIOS.  Since there is no DNS server, we must rely on NetBIOS.

_x__	Clear the NetBIOS cache by typing nbtstat –R.  Check the NetBIOS cache by typing nbtstat -c to make sure it is empty.

_x__	Start a Wireshark capture on Hamilton.

__x_	Ping Ottawa by name;  ping Ottawa.  (The command is not case sensitive).

_x__	Stop the Wireshark capture.

3.	Capture the Wireshark Display.  It should look the same as

figure 2.

Fig. 2 Wireshark capture when Hamilton pings Ottawa by name.

__x_	Display the NetBIOS cache on Hamilton;  nbtstat –c.  There should be an entry for Ottawa.

4.	Capture the NetBIOS cache on Hamilton showing that the name Ottawa has been mapped to an IP address.

This is the explanation of the frames in figure 2.

Frame #1  	Hamilton (10.1.1.3) broadcasts (10.1.1.255) to find the IP address

of the computer called Ottawa.  This is an NBNS (NetBIOS Name

Service) frame.

Frame #2	Ottawa ARPs to find out the MAC address of Hamilton.

Frame #3	Hamilton sends an ARP telling Ottawa what its MAC address is.

Frame #4	Ottawa sends a NetBIOS reply telling Hamilton what its IP address

is.

Frame #5	Hamilton asks for Ottawa’s MAC address.

Frame #6 	Ottawa sends Hamilton its MAC address.

Frames #7 through #14	The pings go through successfully.

_x__	Ping Ottawa by name, again.

This will ensure the mapping for Ottawa to 10.1.1.2 is still found in the NetBIOS cache.  It also ensures the 	MAC to IP mapping is still in the ARP cache.

_x__	Start a new Wireshark capture on Hamilton.

_x__	Ping Ottawa from Hamilton, again.

_x__	Stop the Wireshark capture when the pings have finished.

5.	Capture Wireshark display showing only pings are generated.

6.	Explain why there is no ARP frames or NetBIOS frames generated this time.

NetBIOS uses the following methods to map a name into an IP address.

NetBIOS		    WINS	        broadcast	      lmhosts

cache		    server					file

Fig. 3  4 steps NetBIOS will try when it attempts to translate a name into an IP

We saw how NetBIOS broadcasted to find out Ottawa’s IP address.  When we pinged again, we saw there was no NetBIOS request to find out Ottawa’s IP address because after the first ping, Ottawa’s IP address was added to the NetBIOS cache.   The second time we pinged, Hamilton found Ottawa’s IP address in the NetBIOS cache and it’s MAC address in the ARP cache.

We still have to investigate how the WINS server works.

_x__	On Hamilton, type ipconfig /all.

Notice there is no WINS server configured.  We currently do not have a WINS server running.  Let’s install WINS on Calgary.

Calgary:

_x__	Select Server Manager as shown in figure 4.

Fig. 4  Starting the Server Manager

_x__	Click on Features in the left pane.  Then click on Add Features in the 	right pane.

_x__	Scroll to the bottom of the list and check the box opposite WINS server.

__x_	Finish installing WINS on Calgary.

Hamilton:

_x__	On Hamilton, configure Calgary (10.1.1.10) as the WINS server.  See

figure 5.

__x_	In figure 5, the gateway shows up as 10.1.1.2.  If you have an address for 	the gateway, remove it.  There should be no gateway.

Fig. 5  Configuring Hamilton so it uses Calgary as the WINS server

_x__	On Hamilton, type ipconfig /all.

The primary WINS server should show up as 10.1.1.10 (Calgary’s IP).

_x__	Clear the NetBIOS cache on Hamilton.

_x__	Start a capture in Wireshark.

_x__	Ping Ottawa from Hamilton.

__x_	Stop the Wireshark capture.

7.	Capture the Wireshark display.  It should look like figure 6.

Fig. 6  The result of pinging Ottawa from Hamilton when Hamilton has a WINS

server configured.

According to the flow chart on page 4 of this lab, Hamilton would have checked its NetBIOS cache first.  Having found no entry for Ottawa (since we cleared the NetBIOS cache), the flow chart says step 2 is contact the WINS server.  Here’s an explanation of the frames in figure 6.

Frame #1  	Hamilton wants to contact the WINS server so it ARPs for the MAC address of the WINS server.

Frame #2	Calgary, the WINS server sends an ARP reply.  Hamilton now

knows the MAC address of Calgary.

Frame #3	Hamilton (10.1.1.3) sends a name resolution request to the WINS server, Calgary at 10.1.1.10.

Frame #4	Calgary ARPs to get Hamilton’s MAC address

Frame #5  	Hamilton sends Calgary an ARP reply so Calgary now knows

Hamilton’s MAC address.

Frame #6  	Calgary tells Hamilton it does not know the IP address of Ottawa.  We haven’t configured Ottawa to point to a WINS server so it did not register its WINS record with Calgary.

Frame #7	Hamilton broadcasts on 10.1.1.255 to find out if Ottawa exists on the same segment.

Frame #8	Ottawa ARPs to get Hamilton’s MAC address.

Frame #9	Hamilton sends an ARP reply to tell Ottawa what its MAC address

is.

Frame #10  	Ottawa sends Hamilton its IP address.

Frame #11	Hamilton ARPs to get Ottawa’s MAC.

Frame#12	Ottawa sends its MAC to Hamilton.

The rest of the frames are Hamilton successfully pinging Ottawa.

Let’s take a look at WINS on Calgary.

Calgary:

_x__	On Calgary, click on Start, Administrative Tools, and WINS at the bottom of the list.

__x_	Expand Calgary (10.1.1.10).  Right-click on Active Registrations and select Display Records.  Click on the Find now button.  Figure 7 should be visible.

Fig. 7  The NetBIOS records found in the WINS server

Figure 7 shows a list of all the NetBIOS records found on the WINS server.

_x__	Configure the WINS page of the properties of the NIC on Calgary so it points to itself as the WINS server.  If there are any entries in the WINS box, remove them so only the 10.1.1.10 IP address is left.

Ottawa:

_x__	Configure Ottawa’s NIC to point to Calgary as the WINS server.  You may find an IP address for 10.10.0.2 already on the WINS page.  Delete it.

_x__	Force Ottawa and Calgary to register their NetBIOS records with the WINS server.  To do this type nbtstat –RR at a DOS prompt on both machines.

Calgary:

___	On Calgary, in the WINS snap-in, right-click on Calgary (10.1.1.10) and select Refresh.  Then display the records again.  The display should look like figure 8.

Fig. 8  WINS on Calgary showing the proper records has registered

8.	Capture the WINS display.  It should look like figure 8.

Hamilton:

___	Clear the NetBIOS cache on Hamilton.  (nbtstat –R)

___	Start a Wireshark capture on Hamilton.

___	Ping Ottawa by name.

___	Stop the Wireshark capture.  The display should look like figure 9.

Fig. 9  Hamilton obtains the IP address of Ottawa from the WINS server.

9.	Explain the sequence of events in figure 9.  Provide an explanation each of the first 12 frames.  Use computer names instead of IP addresses.

We have seen how the first 3 blocks of the flowchart work.  The last remaining

block is the configuring of the lmhosts file.  The lmhosts file is used to statically

configure host to IP mappings.  This is useful if you do not have a WINS server

available in your network, yet you do have multiple networks that will not allow

hosts to be found with broadcasts.

Ottawa:

___	Configure the NIC so the gateway address is 10.1.1.10, Calgary’s address.

Normally, you would point to the router but we don’t have one and we need a

gateway address configured to perform the next steps.

___	Open the Windows Explorer.  Navigate to the c:\windows\system32\drivers\etc folder.

___	Use Notepad to open the lmhosts.sam file.

Let’s pretend we have a computer called Japan with a network address of

20.20.20.20.  Since this is a different network than the one Ottawa is on, we

cannot use a broadcast.  Our WINS server does not have a record for it.  We’ll

put a record in the lmhosts file to allow Ottawa to find Japan.

___	Add the following entry to the bottom of the lmhosts file:

20.20.20.20       Japan

Fig. 10  The lmhosts file after the record for Japan has been added

10.	Capture the lmhosts file showing the recorded you added for Japan.

___ 	Save the file as lmhosts without an extension.  Notepad will try to add an extension but you can prevent this by placing quotes around the name lmhosts.   Save the file as “lmhosts” including the quotes.

___	Clear the NetBIOS cache on Ottawa.

Hamilton:

___	Start a fresh Wireshark capture on Hamilton.

Ottawa:

___	Ping Japan.

Hamilton:

___	Stop the Wireshark capture.

Fig. 11  Pinging Japan from Ottawa.  Japan is in the lmhosts file.

11.	Capture the output of the Wireshark display when Ottawa tries to ping Japan.

Here’s the explanation of what happened this time.

Frame #1  	Ottawa wants to contact the WINS server so it ARPs for the MAC address of the WINS server.

Frame #2	Calgary, the WINS server sends an ARP reply.  Ottawa now

knows the MAC address of Calgary.

Frame #3	Ottawa (10.1.1.2) sends a name resolution request to the WINS server, Calgary at 10.1.1.10.

Frame #4	Calgary ARPs to get Ottawa’s MAC address

Frame #5  	Ottawa sends Calgary an ARP reply so Calgary now knows

Ottawa’s MAC address.

Frame #6  	Calgary tells Ottawa it does not know the IP address of Japan.

Frame #7	Ottawa broadcasts on 10.1.1.255 to find out if Japan exists on the same segment.

Frame #8	Ottawa doesn’t get a reply so it broadcasts again just in case the first broadcast got destroyed.

Frame #9	Ottawa doesn’t get a reply so it broadcasts one more time

Frame #10 – 13 Ottawa has found the IP address of Japan in the lmhosts file.  It

realizes Japan is on another network so Ottawa now wants to send the packet to its gateway at 10.1.1.10.  (We configured Ottawa with a gateway that points to Calgary).  Ottawa sends the pings to 20.20.20.20 by passing them to the gateway.

You’re all done