Lab 9		Name _________________________________________

MCSE 1			 Network Printing

Objective:

In this lab you will:

-  install a networked printer

-  install printer drivers

-  configure a print server

-  assign printer permissions

-  configure printer pooling

-  redirect jobs from a failed printer to another printer

Procedure:

Sharing a local printer

In this part of the lab we will pretend we have a small peer-to-peer network like you might have at home.  If you have one printer, you can connect it directly to one computer with a USB cable and share it.  You can then print to this shared printer from any computer in your home network.

___	Start Ottawa (Windows 7) and Hamilton (Windows 8).  Log into the local Administrator’s account on each computer.

___	Make sure you can ping by name between Ottawa and Hamilton.

Ottawa:

___	On Ottawa, click on Start and Devices and Printers.  On the top menu bar click on Add a printer.

___	At the first screen select Add a local printer.

___	On the  Choose a printer port page, select the parallel port, LPT1:(Printer Port).

With today’s printers, you would not be plugging a printer into a parallel port.  You would more than likely use a USB connection.  Most USB printers are installed with the installation disk that accompanies the printer.  We want to install a local printer for which we do not have an installation disk so we will have to pretend it is a parallel port printer.

___	In the Install the printer driver select an HP Laserjet 2300L PS printer.

___	For the printer name, use your last name.

___	On the Printer Sharing page, share the printer using the default Share name which should be your last name.

___	Clear the check box set as the default printer

___	Finish installing the printer.

1.	Capture the window showing the new printer that bears your last name.

___	Right-click on the new printer and set it as the default printer.

2.	How can you tell which printer is the default printer?

Let’s install a printer on Hamilton that sends print jobs to the printer on Ottawa.

Hamilton:

In an earlier lab we configured the option to manage network passwords using

the credentials of  Eng1 on Ottawa.  Let’s remove those credentials.

___	Log into Hamilton as the Lab8 user.

___	Open the control panel.  Select User Accounts, then Manage Windows Credentials.  Click on Remove.  See figure 1 on the next page.

___	Close all the windows.

Let’s add the printer to Hamilton.

___	Log into Hamilton as the Administrator of Hamilton.

___	Open the control panel.  Click on  View devices and Printers.

___	In the menu bar at the top of the window, click on Add a printer.

___	Click on The printer that I want isn’t listed.  See figure 2

Fig. 1  Click on Remove  to get rid of the Eng1 credentials

Fig. 2 Click on The printer that I want isn’t listed

___	Configure the next page as shown in figure 3.  Use the name of the shared printer on Ottawa in place of Clairmont

Fig. 3  Pointing the new printer on Hamilton to the printer on Ottawa

___	Click on Next.  It will take a few minutes while it goes and finds the drivers on Ottawa to load onto Hamilton.

___	On the next page, make sure the box opposite Set as the default printer is checked.  Click on Finish.

3.	Capture the window showing the new printer pointing to Ottawa.

___	Right-click on the printer on Hamilton.  Click on Printer Properties.  Click on the Print test page button on the General page of the printer.

___	Open the printer queue on Hamilton by double-clicking on the printer and then clicking on See what’s printing.

4.	Capture the printer queue on Hamilton showing the print job waiting to be printed.  Make sure all the columns are expanded so you can read what’s in each column.

5.	Capture the printer queue on Ottawa showing the print job waiting

to be printed.

6.	On Ottawa, open the print queue folder (c:\windows\system32\spool\printer).  Capture the view in the Explorer showing the two files that make up the print job.

___	Open the print queue folder on Hamilton.  Notice there are no files in the print queue folder.  They were sent to the print server, Ottawa.

Hamilton:

___	Log out on Hamilton and log back in as the Lab 8 user.

___	Check the Printers and Faxes folder.

7.	Is the network printer the Administrator installed Hamilton, available for the Lab 8 user?

Ottawa:

___	Log out of Ottawa and log back in as arch1.

___	Open the Devices and Printers folder.

8.	Is the local printer the Administrator installed Ottawa, available for the arch1 user?

9.	What can you deduce from the answers in #7 and #8?

It is possible to control who has access to the shared printer by changing the

DACL of the printer.  Let’s try it.

___	Log into Ottawa as the Administrator.

___	Open the Printer Properties pages of the printer you created.

___	click on the Security tab.

___	Remove the group Everyone.

Hamilton:

___	Make sure you are still logged in as the Lab8 user.

___	Open the Devices and Printers folder on Hamilton.

___	Try adding the printer on Ottawa, to Hamilton.

Notice access to the printer on Ottawa is “Denied” now that the group Everyone

has been removed from the local printer on Ottawa.

10.	Capture the Printers and Faxes folder on Hamilton showing the message that says “Windows cannot connect to the printer”

___	Log in as the Administrator.

___	Remove the printer from Hamilton.

Ottawa:

___	Log in as the Administrator.

___	Remove the printer from Ottawa.

Networked Printers

It is becoming more common to find printers containing NICs.  This allows them to be connected to the Ethernet so any computer can be configured to send print jobs to them.  Networked printers add great flexibility to print services.  You are no longer restricted to placing the printer within reach of a USB cable or parallel cable since the printer does not have to be directly connected to a computer.

There are two ways to configure network printing employing a networked printer:

1.	each computer can print directly to the printer

2.	use a printer server that prints to the printer and all other computers send

print jobs to the print server.

Since using a print server affords the greatest number of advantages, we will configure our network to use a print server.  Ottawa will be the print server as it was in the first part of this lab.  Hamilton will send print jobs to it.

___	Make sure you are logged in as the Administrator of Hamilton and Ottawa.

Let’s assume we have configured an IP address of 10.1.1.55/24 on our networked printer.

Ottawa:

___	Click on Start and select Devices and Printers and add a new printer.

___	Configure the printer as shown in the following screen captures.

Select this option

Select this option

Fig. 4  Configuring a networked printer on a Windows 7 computer

Select this option

Normally, you would leave this box checked but we don’t actually have a printer at 10.1.1.55 so we must clear it.

Select this option

Fig. 5  Configuring a networked printer on a Windows 7 computer

Select this printer

Fig. 6  Configuring a networked printer on a Windows 7 computer

___	Set the printer as the default printer.

___	Don’t print a test page.

Hamilton:

___	Add a network printer the way you did in the first part of the lab.  This time point the printer to the net printer on Ottawa.

11.	Capture the Printers and Faxes folder on Hamilton showing the printer is pointing to the “net printer on Ottawa”.

___	Print a test page on Hamilton.

12.	Capture the print queue in the printer on Ottawa.

Printer Pooling

Printer pooling allows one installed printer to point to a number of actual printers.

The purpose is to speed up printing in an environment where there is a lot of

printing by multiple users, such as a computer lab at a college or university.

Ottawa:

___	Add another network printer on IP address 10.1.1.56.

___	Make sure you select the same printer; HP Laserjet 2300 ps.

___	The printer name will be net printer 2.

___	Set the printer as default.

___	Don’t bother printing a test page.

13.	Capture the Devices and Printers folder showing the 2 printers.

Let’s configure printer pooling.

___	Open the printer properties page of net printer.  Click on the Ports tab.

___	Click on Enable printer pooling.

Fig. 9  Adding a second port for printer pooling

___	Click on the second printer 10.1.1.56.

Now when the user sends a print job to the net printer, if the printer is busy the

job will be sent to the net printer 2 printer.

14.	Capture the Ports page of printer1 showing that printer pooling has been configured.

Redirecting print jobs on a failed printer

Ottawa:

___	Deactivate the printer pooling option on the net printer.

A printer might fail while people are still sending print jobs to it.  The print jobs will

build up in the queue.  The Administrator can configure a printer to send print

jobs from a failed printer to another printer that is still working.

Let’s assume net printer has malfunctioned and will not print anymore.  We want

to redirect any print jobs sent to net printer to net printer 2.

___	Open the properties page of net printer.  Click on the Ports tab.  Click on Add Port.

___	Highlight Local Port and click on the New Port button.

___	For the Port Name use \\Ottawa\net printer2.

15.	Capture the Ports page of printer1 showing that print jobs are now being redirected to printer2.

Hamilton:

___	Print two test pages from net printer.

___	Open the printer queue on printer2.  You should see two “down-level” documents waiting to be printed.

16.	Capture the printer queue on net printer 2 showing the

“Remote downlevel Documents” waiting to be printed.

Give up and go home !